# Client Interface:

package BioNex;

public interface HiveClientInterface

{

void Init(String host, int port) throws HiveClientException;

void AddWorkSet(String workset_xml) throws HiveClientException;

void SetPlateFate(String barcode, String fate) throws HiveClientException;

void EndBatch(String fate) throws HiveClientException;

void Ping() throws HiveClientException;

}

## Summary:

The intent of the HiveClient interface is to wrap and hide the details of the xml-rpc interface used to communicate from GEMS to the BioNex Hive system.

GEMS will need to either create several HiveClient objects (one per unique Hive system) or re-use the HiveClient object by calling the Init() method before every call to an action method.

## Method descriptions:

### 1. Init(String host, int port)

Allocates a new Client connection.  Only needs to be called once per application lifetime, repeat calls will discard the current connection and start a new one.

Sets the remote host and port to connect to.

Host can be a resolvable machine name or ip-address, should be the machine that synapsis is running on.

The hive will default to listen on port 5678, but this is configurable

### 2. AddWorkSet(String workset_xml)

Transfers a workset from GEMS to Synapsis.  The workset is appended to Synapsis’ work queue and will be processed in order.  There is no provision for removing or re-ordering worksets via GEMS.  These will be local terminal operations in Synapsis.  The workset xml format will be documented separately.

### 3. SetPlateFate(String barcode, String fate) throws HiveClientException;

Sets the fate string for a specific barcode/plate.

There is currently no validation done on either the received barcode or fate strings.

If there is no exact match to the received barcode, the fate for that barcode is accepted, but no action will result.

### 4. EndBatch(String fate)

Tells Synapsis that no more plates are expected to join the named fate, so Synapsis is free to unlock the cart to which those plates have been assigned.

### 5. Ping()

Do-nothing method to aid in determining if there is a listening server on the other side of the xml-rpc connection.

### Implementation class is

Package BioNex

public class HiveClient implements HiveClientInterface

{}

use the default constructor.

-------------------------------------------------------------------------

# Server Interface:

package BioNex;

public interface HiveServerInterface

{

void Init(int port, HiveServerCallbackInterface callbacks);

void Start() throws HiveServerException;

void Stop();

}

requires callback interface:

package BioNex;

public interface HiveServerCallbackInterface

{

Void DestinationPlateComplete(String hive_name, String desination_barcode, TransferMap[] mapping);

void ReinventoryComplete(String hive_name, String cart_name, Object[] barcodes);

void Ping();

}

public class TransferMap

{

public String source_barcode;

public int source_row;

public int source_column;

public int destination_row;

public int destination_column;

public double transfer_volume;

}

## Summary:

The Intent of the HiveServer interface is to wrap and hide the details of the xml-rpc interface provided by GEMS for the Hive systems to call in to.  The intention is that the GEMS side of the connection will instantiate ONE HiveServer object to service the calls of all Hive systems.

## HiveServer Method Descriptions:

### 1. Init(int port, HiveServerCallbackInterface callbacks)

Saves the port and hands a callback object to the internal object responsible for handling xml-rpc calls.

The Hive defaults to using port 6789, but is configurable.

The methods on the callback interface are called in response to remote method calls.

The callback interface implementation is intended to be provided by the TPS team at Monsanto.

### 2. Start()

Starts the xml-rpc web listener in a new thread.  Multiple calls to Start with no call to Shutdown result in undefined behavior.

### 3. Shutdown()

Stops the xml-rpc web listener and ends the thread.

Implementation class is

package BioNex;

public class HiveServer implements HiveServerInterface

{}

use the default constructor

## HiveServerCallbackInterface method Descriptions:

### 1. DestinationPlateComplete(String hive_name, String destination_barcode, TransferMap[] mapping)

Called when a remote Hive system completes processing a destination plate (the plate has arrived at its destination cart).  The TransferMap[] mapping is an array of transfers which specify the source barcode, source well, and volume transferred for each well of the destination plate that was filled by the hive.  There is no record for unmodified destination wells.

### 2. ReinventoryComplete(String hive_name, String cart_name, String[] barcodes)

Called when a remote Hive system completes an inventory on a cart.

The Object[] barcodes is an array of strings representing the contents of the cart.

Newly inventoried plates are automatically inserted into "Hold" fate until GEMS explicitly sets their fates.

### 3. Ping()

Called when a remote Hive system checks to see if the server is responding to xml-rpc requests.

There is no need to do anything in this method, it's provided for debugging purposes.